Audio MCU Development Board Specification

- OVERVIEW

The Audio MCU Development Board is a custom embedded hardware platform intended for real-time audio and guitar-effects development.

The board will combine a microcontroller, analog audio input/output circuitry, ADC/DAC conversion, power regulation, and USB programming/debug capability onto a single PCB. The primary goal is to create a reuseable and convenient platform for experimenting with digital audio processing.

The board should accept a standard ¼ inch instrument-level input, convert the signal into a form suitable for digital processing by the MCU, and reconstruct an analog signal at a ¼ inch output.

- PRIMARY DESIGN GOALS

- Accept a standard guitar/instrument signal through a 1/4 inch TS input.

- Present a high input impedance appropriate for passive guitar pickups

- Protect the analog front end and ADC from ESD, transients, and signals outside the allowable voltage range.

- Provide analog buffering and appropriate gain before conversion.

- Bias bipolar audio appropriately for a single-supply ADC.

- Provide analog anti-alias filtering.

- Digitize audio at a rate and resolution appropriate for guitar/audio DSP.

- Process audio in real time using an onboard MCU.

- Convert processed audio back to analog.

- Filter and buffer the analog output.

- Provide a standard ¼ inch TS output.

- Operate from a conventional 9v pedal-style DC supply.

- Provide onboard regulation for analog and digital supply rails.

- Support programming/debugging through USB.

- Expose useful MCU pins for experimentation/expansion

- Provide basic user controls such as potentiometers, switches, buttons, LEDs, etc.

- SPECIFICATIONS

- Audio input: 1 x mono ¼” TS

- Audio output: 1 x mono ¼” TS

- Input impedance: > 1M

- Nominal Source: Guitar/Instrument

- Sample Rate: 48 kHz

- ADC resolution: 16 – 24 bit

- Audio Bandwidth: 20 – 20kHz

- Processing: Real time, low latency

- Main power Input: 9v DC

- Programming: USB

- Debug: SWD/JTAG as appropriate

- Expansion: GPIO/SPI/I2C/UART/Analog

- PCB: Custom MCU and analog circuity, 4 layer preferred

3. SYSTEM DIAGRAM

- SCHEMATIC LAYOUT